# Specialized Experiment 2

## Hydrological Modeling: SCS-CN Runoff Calculation

Week 5 Session B | Duration: 2 hours | Smart Water Lab Series

## Experiment Overview

This experiment focuses on implementing the Soil Conservation Service Curve Number (SCS-CN) method, the most widely used approach for estimating direct runoff from rainfall. You will translate the mathematical formula into Python code, handle physical boundary conditions, and perform parameter sensitivity analysis.

## Learning Objectives

•  Translate mathematical formulas into Python code using AI assistance

•  Handle physical boundary conditions (e.g., P < Ia)

•  Perform parameter sensitivity analysis

•  Validate AI-generated code with domain knowledge

•  Create visualization of runoff behavior

## Physical Background

### The SCS-CN Formula

Runoff Calculation Formula:
Q = (P - Ia)² / (P - Ia + S)
Where:
- Q = Runoff depth (mm)
- P = Rainfall depth (mm)
- S = Potential maximum retention (mm)
- Ia = Initial abstraction (mm) = 0.2 × S
S Formula:
S = (25400 / CN) - 254
CN = Curve Number (0-100)

### Curve Number (CN) Values

| Land Use / Condition | Typical CN Value |
| --- | --- |
| Woods, good condition | 60-70 |
| Pasture, fair condition | 75-85 |
| Cultivated, straight row | 80-90 |
| Urban, residential | 75-95 |
| Paved areas | 95-100 |

### Physical Boundary Conditions

•  If P < Ia: No runoff occurs, Q = 0

•  If CN = 100: Impervious surface, maximum runoff

•  If CN = 0: All water infiltrates, Q = 0

•  Runoff cannot exceed rainfall: Q ≤ P

## Prerequisites

•  Completed Experiment 1 (Rainfall Alert System)

•  Understanding of basic hydrology concepts

•  Familiarity with Python functions and NumPy

•  Knowledge of matplotlib for visualization

## Materials and Tools

•  OpenCode Cloud IDE

•  Python 3.10+

•  Required libraries: numpy, matplotlib, pandas

•  Sample rainfall data for testing

## Experiment Structure

### Part 1: Formula Implementation (30 min)

Task: Use AI to implement the SCS-CN formula in Python.

Sample Prompt:
I am implementing the SCS-CN runoff calculation method. Please write a Python function calculate_runoff(P, CN) that:
1. Calculates S using: S = (25400 / CN) - 254
2. Calculates Ia = 0.2 * S
3. Returns Q = 0 if P < Ia
4. Otherwise returns Q = (P - Ia)² / (P - Ia + S)
5. Ensures Q never exceeds P
Include docstring and type hints.

### Part 2: Boundary Condition Testing (25 min)

Task: Write comprehensive tests for boundary conditions.

Test Cases to Implement:
1. P = 0: Expected Q = 0
2. P < Ia: Expected Q = 0
3. P = Ia: Expected Q = 0
4. Normal case: P = 50mm, CN = 80
5. Maximum CN: CN = 100
6. Verify Q ≤ P for all cases

### Part 3: Sensitivity Analysis (35 min)

Task: Analyze how runoff changes with different CN values.

Sensitivity Analysis Requirements:
1. Fix P = 50mm
2. Calculate Q for CN = [60, 70, 80, 90, 95, 100]
3. Create a line plot: CN vs Q
4. Create a comparison plot: Rainfall vs Runoff for different CN values
5. Document observations about the relationship

### Part 4: Validation & Documentation (30 min)

Task: Validate physical correctness and document findings.

1. Verify all boundary conditions pass

2. Confirm Q ≤ P always holds

3. Check that higher CN produces more runoff

4. Document any AI errors found and corrections made

5. Update Prompt Log with all AI interactions

## Deliverables

•  scscn_runoff.py - Main implementation with calculate_runoff() function

•  test_scscn.py - Comprehensive test suite

•  sensitivity_analysis.py - Visualization code

•  runoff_comparison.png - Generated plot comparing CN values

•  prompt_log.md - Documentation of AI interactions

## Grading Criteria

| Criterion | Points | Weight |
| --- | --- | --- |
| Formula Implementation (correct, handles boundaries) | 25 | 25% |
| Test Suite (comprehensive, all pass) | 25 | 25% |
| Sensitivity Analysis (plots, observations) | 25 | 25% |
| Physical Validation (all constraints verified) | 15 | 15% |
| Prompt Log (documented AI interactions) | 10 | 10% |

## Expected Results

For P = 50mm, CN = 80:
S = (25400 / 80) - 254 = 63.5 mm
Ia = 0.2 * 63.5 = 12.7 mm
Q = (50 - 12.7)² / (50 - 12.7 + 63.5)
Q = 1391.29 / 100.8 = 13.8 mm
Verification: 13.8 ≤ 50 ✓

## Tips for Success

• Always verify the formula implementation with known examples
• Test edge cases first — they often reveal bugs
• Use numpy for vectorized calculations (faster for large datasets)
• Physical validation is your best defense against AI errors
• If AI output looks wrong, it probably is — trust your domain knowledge

## Optional Extensions

•  Implement time-area method for watershed routing

•  Add antecedent moisture condition (AMC) adjustments

•  Create interactive plot with sliders for P and CN

•  Compare SCS-CN with other runoff methods (Rational method)