# Specialized Experiment 3

## Water Resources Optimization: Reservoir Dispatch

Week 6 Session A | Duration: 2 hours | Smart Water Lab Series

## Experiment Overview

This experiment focuses on solving a multi-objective optimization problem for reservoir management. During drought periods, a reservoir must balance two competing objectives: maintaining downstream ecological flow while maximizing storage for hydropower generation. You will formulate the optimization problem, implement it using scipy.optimize, and analyze the trade-offs between competing objectives.

## Learning Objectives

•  Formulate multi-objective optimization problems

•  Use scipy.optimize for constrained optimization

•  Analyze trade-offs between competing objectives

•  Generate optimal release schedules

•  Validate constraints are satisfied

## Problem Statement

### Scenario

A reservoir with the following characteristics must optimize water release over a 7-day period during a drought:

Reservoir Parameters:
- Current Storage: 500,000 m³
- Minimum Storage (V_min): 100,000 m³
- Maximum Storage (V_max): 1,000,000 m³
- Minimum Ecological Release (Q_eco): 10 m³/s
- Maximum Release (Q_max): 100 m³/s
- Inflow Forecast: [15, 12, 10, 8, 12, 15, 18] m³/s
- Hydropower Price: [0.08, 0.08, 0.08, 0.08, 0.10, 0.12, 0.10] $/kWh

### Objectives

•  Maximize: Hydropower revenue (release × price)

•  Minimize: Ecological deficit (violations of minimum release)

### Constraints

Physical Constraints:
1. Storage bounds: V_min ≤ V_storage ≤ V_max
2. Release bounds: Q_eco ≤ Q_release ≤ Q_max
3. Storage balance: V_t+1 = V_t + (Inflow - Release) × Δt

## Prerequisites

•  Completed Experiments 1 and 2

•  Understanding of basic optimization concepts

•  Familiarity with Python functions and NumPy

•  Knowledge of scipy.optimize basics

## Materials and Tools

•  OpenCode Cloud IDE

•  Python 3.10+

•  Required libraries: numpy, scipy, matplotlib

## Experiment Structure

### Part 1: Problem Formulation (25 min)

Task: Define the optimization problem mathematically.

Formulation Steps:
1. Define decision variables: Q_release for each day (7 variables)
2. Define objective function: Maximize total revenue
3. Define constraints: Storage bounds, release bounds, mass balance
4. Use AI to help write the mathematical formulation

### Part 2: Implementation (35 min)

Task: Implement the optimization using scipy.optimize.

Sample Prompt:
I need to solve a reservoir optimization problem using scipy.optimize. The problem has:
- 7 decision variables (daily releases)
- Objective: maximize hydropower revenue
- Constraints: storage bounds, release bounds, mass balance
Please write Python code using scipy.optimize.minimize that:
1. Defines the objective function
2. Sets up all constraints
3. Solves for optimal releases
4. Returns the optimal schedule and total revenue

### Part 3: Trade-off Analysis (30 min)

Task: Analyze the trade-off between hydropower and ecology.

Analysis Tasks:
1. Run optimization with different weights on objectives
2. Create a Pareto frontier plot
3. Document: What happens if we prioritize ecology over revenue?
4. Calculate the cost of maintaining minimum ecological flow

### Part 4: Validation (30 min)

Task: Verify the solution is physically valid.

1. Check all storage values are within bounds

2. Verify all releases meet minimum ecological requirement

3. Confirm mass balance is satisfied each day

4. Validate total revenue calculation

5. Document any constraint violations found

## Deliverables

•  reservoir_optimize.py - Optimization implementation

•  optimal_schedule.csv - 7-day optimal release schedule

•  tradeoff_analysis.png - Pareto frontier plot

•  prompt_log.md - Documentation of AI interactions

•  validation_report.txt - Constraint verification results

## Grading Criteria

| Criterion | Points | Weight |
| --- | --- | --- |
| Problem Formulation (correct math formulation) | 20 | 20% |
| Implementation (working optimization code) | 25 | 25% |
| Optimal Schedule (valid, feasible solution) | 20 | 20% |
| Trade-off Analysis (Pareto plot, insights) | 20 | 20% |
| Validation (all constraints verified) | 10 | 10% |
| Prompt Log (documented AI interactions) | 5 | 5% |

## Expected Results

Sample Optimal Schedule (Day 1-7):
Releases: [12.5, 10.2, 10.0, 10.0, 13.2, 15.8, 18.0] m³/s
Total Revenue: ~$45,000
Ecological Violations: 0
Note: Actual values may vary based on optimization approach

## Tips for Success

• Start with a simple single-day optimization before extending to 7 days
• Use bounds constraints to keep releases physically reasonable
• If optimization fails, check that constraints are feasible
• Visualize storage trajectory to verify mass balance
• AI can help with scipy syntax, but you must verify the formulation

## Optional Extensions

•  Add uncertainty in inflow forecasts

•  Implement rolling horizon optimization

•  Add water quality constraints

•  Compare different optimization algorithms (SLSQP vs L-BFGS-B)