# Specialized Experiment 4

## Flood Inundation Analysis (DEM-based)

Week 6 Session B | Duration: 2 hours | Smart Water Lab Series

## Experiment Overview

This experiment focuses on analyzing flood inundation using Digital Elevation Model (DEM) data. You will implement a spatial comparison algorithm to identify flooded areas based on water level, create visual flood extent maps, and calculate flooded area percentages. This is a fundamental technique used in flood risk assessment and emergency management.

## Learning Objectives

•  Process Digital Elevation Model (DEM) data

•  Implement spatial comparison algorithms

•  Create visual flood extent maps using matplotlib

•  Calculate flooded area percentages

•  Validate that flooded area increases with water level

## Physical Background

### Digital Elevation Models (DEM)

A DEM is a 2D grid where each cell contains an elevation value (in meters). Common sources include USGS SRTM (30m resolution) and ALOS PALSAR (12.5m resolution).

### Flood Inundation Logic

Flooding Condition:
A location is FLOODED if:
Elevation < Flood_Water_Level
Inundation Depth:
Depth = Flood_Water_Level - Elevation (if flooded)
Depth = 0 (if not flooded)
Flooded Area Percentage:
% = (Number of flooded cells / Total cells) × 100

## Prerequisites

•  Completed Experiments 1, 2, and 3

•  Understanding of NumPy array operations

•  Familiarity with matplotlib for visualization

•  Basic knowledge of spatial data concepts

## Materials and Tools

•  OpenCode Cloud IDE

•  Python 3.10+

•  Required libraries: numpy, matplotlib

•  Sample DEM data (generated or real)

## Experiment Structure

### Part 1: DEM Data Preparation (20 min)

Task: Create or load DEM data for analysis.

Option 1: Generate Synthetic DEM
Create a 100×100 grid with elevation values between 30-80m:
- Use numpy.random for random terrain
- Or create a simple slope pattern
Option 2: Load Real DEM Data
Download sample DEM from:
- USGS Earth Explorer
- OpenTopography

### Part 2: Flood Simulation (30 min)

Task: Implement flood inundation calculation.

Sample Prompt:
I have a 2D numpy array 'dem' representing terrain elevation (100×100). Please write Python code to:
1. Simulate flooding at a given water level
2. Create a boolean mask for flooded cells (elevation < water_level)
3. Calculate inundation depth for flooded cells
4. Calculate flooded area percentage
5. Return the mask, depth array, and percentage

### Part 3: Visualization (35 min)

Task: Create flood extent visualizations.

Visualization Requirements:
1. Original DEM as grayscale image
2. Flood extent as blue overlay
3. Inundation depth heatmap (blue color scheme)
4. Side-by-side comparison at different water levels
5. Include colorbar and title

### Part 4: Dynamic Simulation (25 min)

Task: Simulate rising water levels and analyze trends.

Dynamic Simulation Tasks:
1. Loop through water levels from 40m to 50m
2. Calculate flooded percentage at each level
3. Create a plot: Water Level vs. Flooded Percentage
4. Verify: Does flooded area increase monotonically?
5. Document any unexpected behavior

### Part 5: Validation (10 min)

Task: Validate physical correctness.

1. Verify flooded area increases with water level

2. Check that maximum depth equals (water_level - min_elevation)

3. Confirm flooded percentage is between 0-100%

4. Validate edge cases (water below min elevation, above max elevation)

## Deliverables

•  flood_inundation.py - Main implementation

•  dem_data.npy - DEM data file (or generation script)

•  flood_extent_40m.png - Visualization at 40m water level

•  flood_extent_50m.png - Visualization at 50m water level

•  flood_curve.png - Water level vs. flooded percentage plot

•  prompt_log.md - Documentation of AI interactions

## Grading Criteria

| Criterion | Points | Weight |
| --- | --- | --- |
| DEM Data (loaded or generated correctly) | 15 | 15% |
| Flood Calculation (correct logic) | 25 | 25% |
| Visualization (clear, informative plots) | 25 | 25% |
| Dynamic Simulation (working, validated) | 20 | 20% |
| Physical Validation (all checks pass) | 10 | 10% |
| Prompt Log (documented AI interactions) | 5 | 5% |

## Sample Code Structure

# flood_inundation.py - Structure
import numpy as np
import matplotlib.pyplot as plt
def load_dem(filepath=None):
    # Load or generate DEM data
    pass
def calculate_flood(dem, water_level):
    # AI-generated: flood calculation
    # Returns: flooded_mask, depth_array, percentage
    pass
def visualize_flood(dem, flooded_mask, depth):
    # Create visualization
    pass
def simulate_rising_water(dem, levels):
    # Dynamic simulation
    pass
if __name__ == '__main__':
    dem = load_dem()
    # Run analysis...

## Tips for Success

• Start with synthetic DEM data for predictable results
• Use matplotlib's imshow() for quick visualization
• Blue colormap ('Blues') works well for water visualization
• Physical validation: flooded area MUST increase with water level
• If results look wrong, check your elevation units (meters vs. feet)

## Optional Extensions

•  Load and analyze real DEM data for a specific watershed

•  Implement flood routing (water spreads to adjacent cells)

•  Add building footprints as barriers to flooding

•  Create animated GIF of rising water levels

•  Calculate flood volume (depth × cell area × count)